Assignment 8: VLAN and Inter VLAN Routing.

- Explain Switch?

Ans: A switch is a networking device used in a Local Area Network (LAN) to connect multiple devices such as computers, printers, and servers. It operates mainly on layer 2 (Data Link Layer) of the OSI model.

- Explain Switch Boot Sequence?

Ans: - When the switch is powered on, it performs POST to check hardware.

-The Boot Loader stored in ROM is executed.

-The Boot Loader loads the IOS from the flash memory into RAM.

-The IOS loads the startup configuration from NVRAM.

-The switch initializes interfaces and becomes ready to forward data.

- Explain Three Methods to access Switch Command Line Interface?

Ans: The three methods are:

- Console Port Access: It is the direct physical connection using a console cable through RJ-45 to USB/Serial. It is also used for initial configuration. Additionally it requires terminal software like PUTTY

- Telnet Access: It is a Remote access over a network. It requires IP address configuration on the switch. There is no security.

- SSH Access: The Secure Shell is the secure remote access over a network. It encrypts username, password, and data. It is the preferred method for remote management.

- Explain and Configuring the Cisco Internet Operating System?

Ans: Cisco IOS is the network operating system used in Cisco routers and switches.
It manages:

- Interface configuration

- VLAN and routing setup

- Security settings

- Network protocols

- Device management

IOS provides a Command Line Interface (CLI) for configuration and monitoring.

- Cisco IOS Modes:

  - User Exec Mode

  - Privileged Exec mode

  - Global config mode

  - Interface configuration mode.

5. Explain Switch Port

Ans: A switch port is a physical interface on a network switch used to connect devices such as computers, printers, servers, or other switches. Each port has a unique port number (e.g., FastEthernet0/1, GigabitEthernet0/1). It operates at Layer 2 (Data Link Layer). It sends and receives data frames using MAC addresses. Ports can be configured in different modes: Access Port → Connects end devices (belongs to one VLAN). Trunk Port → Connects switches (carries multiple VLANs).

MCQs:

1. 4-R1, R2, R3, and R4 have their Fast Ethernet 0/0 interfaces attached to the same VLAN. A network engineer has typed a configuration for each router by using a word processor. He will later copy and paste the configuration into the routers. Examine the following exhibit, which lists configuration for the four routers, as typed by the network engineer. Assuming that all four routers can ping each other’s LAN IP addresses after the configuration has been applied, choose the routers that will be able to form a neighbor relationship with the other routers on the LAN. (You can assume that, if not shown in the exhibit, all other related parameters are still set to their defaults.) (Choose two):

Ans: R1, R2

2. enable secret [password] is hashed using the__________ algorithm.

Ans: MD5 algorithm

3. An engineer connects to Router R1 and issues a show ip ospf neighbor command. The status of neighbor 2.2.2.2 lists FULL/BDR. What does the BDR mean?

Ans: D. Router 2.2.2.2 is a backup designated router.

4. Which command is used to view the neighbor discovery table on a PC?

Ans: D. netsh interface ipv6 show neighbors

5. What type of variable is being shown? Routers = [R1,R2,R3]?

Ans: A. List.

6. Identify the fields in an IPv4 header. (Choose three)?

Ans: 	B. Time to Live

C. Source address

D. Destination address